# Pre‑Lab – Experiment 1 (ATmega32)
Parts 1.1 and 1.2

Submission note: As requested, the main code below uses the course header ee470avr.h. A portable version without the header is included in the appendix for reference.

## 1.1  Switch → LED Bargraph (Binary Count)

Objective: Count valid presses on PB0 and display the 8‑bit count on a LED bargraph connected to PORTD. Include debouncing on press and release (20 ms).

Initialization summary:

| PB0 | Input (switch) |
| --- | --- |
| PORTD | Outputs to LED bargraph (D7..D0) |
| Debounce | 20 ms on press and release |
| Display | Binary count x on PORTD |

Code:

// switch-bargraph.c
#include "ee470avr.h"

int main(void){
    unsigned char x = 0;

    bitclr(DDRB, PB0);   // PB0 as input (SW)
    DDRD  = 0xFF;        // PORTD as output (bargraph)
    PORTD = 0x00;        // clear bargraph

    while(1) {
        while (bittst(PINB, PB0) == 0);   // wait for switch ON (PB0==1)
        _delay_ms(20);                    // debounce

        x++;
        PORTD = x;                        // display count on PORTD

        while (bittst(PINB, PB0) != 0);   // wait for switch OFF (PB0==0)
        _delay_ms(20);                    // debounce
    }
}

## 1.1 Switch–Bargraph Counter (Flowchart)

## Start
↓
Initialize Ports

## PB0 as input (switch)

## PORTD as output (bargraph)

## Clear PORTD

## ↓
Wait for Switch Pressed
(while PB0 == 0)

## ↓
Debounce Delay (20 ms)

## ↓
Increment Counter (x++)

## ↓
Send Count to Bargraph
(PORTD = x)

## ↓
Wait for Switch Released
(while PB0 != 0)

## ↓
Debounce Delay (20 ms)

## ↓
Repeat Loop

## ↓
End

## 1.2  Count Switch Presses (mod 16) → 7‑Segment (Common Cathode) + Copy PB0 to LED

Objective: On each valid press of PB0, increment a 4‑bit counter b (0…15) and show it on a common‑cathode 7‑segment display on PORTD. Mirror the switch level on LED PB5.

Segment mapping used: PORTD pins drive segments a..g (dp unused) per lab schematic; device is Common Cathode.

Code:

/* count-sw-led-7seg.c

Count switch pressings and directly display on 7-segment

display-Common cathode connected to PORTD.

If the count reaches 15, the next count should be 0 and repeats.

Copy SW on PB0 to LED on PB5.

*/

#include "ee470avr.h"

unsigned char b;

// unsigned char map[16]={0x3f,0x06,0x5b,0x4f,0x66,0x6d,0x7d,

//   0x07,0x7f,0x6f,0x77,0x7c,0x58,0x5e,0x79,0x71};//0..0xF

int main() {

bitclr(DDRB,PB0);   // SW

bitset(DDRB,PB5);   // LED

DDRD=0xFF;          // pins 0 to 6 of PORTD are o/p for 7seg CC

b=0; PORTD=map[b];  // put 0 on 7seg CC on port D

while(1) {

while(bittst(PINB,PB0)==0) {

bitclr(PORTB,PB5);// wait until PB0==1

}

b++;

if (b==16) b=0;

PORTD=map[b];   // write to 7-segment display

_delay_ms(50);  // wait for 50 msec

while (bittst(PINB,PB0)!=0) {

bitset(PORTB,PB5);

} // wait until PB0==0

}

1.2 7-Segment Display Counter (Flowchart)

Start
↓
Initialize Ports

- PB0 as input (switch)

- PORTD as output (7-segment)

- Display digit 0

↓
Wait for Switch Pressed
(while PB0 == 0)

↓
Debounce Delay (20 ms)

↓
Increment Counter (n = n + 1)
If n > 9, reset to 0

↓
Update 7-Segment Display
(PORTD = DIGIT[n])

↓
Wait for Switch Released
(while PB0 != 0)

↓
Debounce Delay (20 ms)

↓
Repeat Loop

↓
End